Battery Storage & Solar Expansion

Investment Analysis for Your 5.985 kW Solar System

Encinitas, CA (92024) | SDG&E EV-TOU-5 Rate Plan | January 2026

# Executive Summary

Based on your actual January 2026 usage data, this analysis evaluates the financial viability of adding battery storage and/or expanding your solar system.

Key Finding: With the federal residential solar tax credit (25D) having expired on December 31, 2025, the economics have shifted. Battery storage now requires 7-10 year payback periods without incentives, though California SGIP rebates may still provide significant savings for qualifying households.

# Your Current System Performance

Analysis based on 25 days of actual data from January 2-26, 2026:

| Metric | Value |
| --- | --- |
| Total Solar Production | 448.8 kWh (18.0 kWh/day avg) |
| Total Grid Consumption | 672.2 kWh (26.9 kWh/day avg) |
| Total Grid Export | 158.9 kWh (6.4 kWh/day avg) |
| On-Peak Usage (4-9pm) | 207.4 kWh (8.3 kWh/day avg) |
| Est. Monthly Grid Cost | $186.21 |
| Est. Annual Grid Cost | $2234.57 |

# Battery Storage Options

The following table compares the top residential battery systems available in 2026:

| System | Capacity | Power | Warranty | Est. Cost |
| --- | --- | --- | --- | --- |
| Tesla Powerwall 3 | 13.5 kWh | 11.5 kW | 10 years | $10,500 - $13,000 |
| FranklinWH aPower 2 | 15 kWh | 10 kW | 15 years | $14,000 - $18,000 |
| Enphase IQ 5P (x3) | 15 kWh | 11.5 kW | 15 years | $15,000 - $20,000 |
| LG RESU16H Prime | 16 kWh | 7 kW | 10 years | $11,000 - $14,000 |

## Battery ROI Analysis for Your Home

Based on your on-peak usage patterns, here is how a battery could provide savings:

- On-Peak Usage: 207.4 kWh over 25 days (8.3 kWh/day average)

- Rate Differential: On-Peak (41.9¢) vs Super Off-Peak (13.3¢) = 28.6¢/kWh savings

- Monthly Potential Savings: $59.32 (shifting all on-peak to super off-peak)

- Annual Potential Savings: $711.87

| Battery Cost | Simple Payback | With SGIP Rebate* |
| --- | --- | --- |
| $10,000 | 14.0 years | 2.1 years |
| $15,000 | 21.1 years | 3.2 years |
| $20,000 | 28.1 years | 4.2 years |

*SGIP Equity Rebate of $850/kWh (if eligible). General Market rebate is lower.

# Available Incentives (2026)

## Federal Tax Credits

Important: The 30% Residential Clean Energy Credit (Section 25D) expired on December 31, 2025. Customer-owned solar and battery systems installed in 2026 are

no longer eligible for federal tax credits.

- Solar/battery leases and PPAs may still qualify through the 48E business credit (claimed by the lessor)

- Commercial installations must begin construction by July 4, 2026 to qualify for credits

## California SGIP (Self-Generation Incentive Program)

SGIP provides state-funded rebates for battery storage in SDG&E territory:

| Category | Rebate Amount | Eligibility |
| --- | --- | --- |
| Equity Resiliency | $1,000/kWh | Low-income + 2+ PSPS events or medical baseline |
| Equity | $850/kWh | Low-income residential |
| General Market | ~$200-400/kWh | All residential customers |

Note: SGIP funding is limited and applications are processed first-come, first-served. Work with an approved SGIP installer to secure your rebate.

# Solar Expansion Options

Your current 5.985 kW system produces an average of 18.5 kWh/day in January. Expanding capacity could help offset more of your grid consumption.

## Considerations for Expansion

- Sunrun Lease/PPA: Since you have an existing Sunrun system, expanding through them may be simplest. Sunrun now offers battery add-ons for existing customers.

- New Installation: Adding a second system from a different provider is possible but adds complexity. Some customers report getting better pricing from local installers.

- Panel Upgrade Costs: Your electrical panel may need upgrading (~$4,000-$5,000), though Sunrun covers up to $4,000 of this cost.

- NEM 3.0 Impact: New solar capacity will be under NEM 3.0, which pays ~75% less for exports. This makes battery storage more valuable for new installations.

## Estimated Costs for Solar Expansion

| Added Capacity | Est. Cost | Added kWh/day | Notes |
| --- | --- | --- | --- |
| +3 kW | $8,000 - $12,000 | ~9 kWh | Modest expansion |
| +5 kW | $12,000 - $18,000 | ~15 kWh | Significant increase |
| +8 kW | $18,000 - $26,000 | ~24 kWh | Near-doubling capacity |

# Recommendations

Best Value Option: If you qualify for SGIP Equity rebates ($850-$1,000/kWh), battery storage becomes highly attractive with payback periods under 2 years. Check your eligibility at sgipsd.org.

## If You Qualify for SGIP Equity Rebates

- Add a 13.5-15 kWh battery (Tesla Powerwall 3 or FranklinWH aPower 2)

- Net cost after rebate: $1,500 - $4,000

- Payback: 1-2 years

## If You DON'T Qualify for SGIP Equity Rebates

- Battery-only: 10-15 year payback makes standalone battery storage less attractive

- Consider waiting for battery prices to drop further or new incentive programs

- Solar expansion through Sunrun lease/PPA (they claim the 48E credit) may still provide value

## Additional Considerations

- Backup Power Value: If you experience frequent outages or PSPS events, battery backup has value beyond pure ROI

- EV Integration: Your EV already charges during super off-peak (12am-6am), which is optimal. A battery could provide additional flexibility.

- Future Rate Changes: SDG&E rates tend to increase 3-5% annually, which improves battery economics over time

Analysis prepared January 2026 | Data source: Your actual SDG&E and Sunrun usage